R65 SYSTEM REFERENCE GUIDE

Monitor Commands	2

Pascal Commands	4

Error Codes	6

Control Keys (Non-printable Characters)	9

File Types	10

How-To — Installation	11

How-To — Working with Disks	12

How-To — Working with Pascal	14

How-To — Printing / printout.txt	15

How-To — Editing Files	17

How-To — Debugging	18

How-To — File Transfer (Host ↔ R65)	19

## Monitor Commands

Monitor commands operate on files, memory, and system control. They are used to control the R65 microprocessor at the machine language level. Programs for this level are written with a 6502 symbolic assembler.

| Command | Arguments | Description | Notes |
| --- | --- | --- | --- |
| CLRB | address | Clear breakpoint at address | — |
| COPY | file, from, to | Copy file between drives | Max size 32 KB |
| DATE | — | Display and modify date | Initialized from host |
| DELETE | file, drive | Delete file | Exact name required |
| DFORMAT | diskname, drive | Format disk directory | Erases directory |
| DIR | drive | Display directory | — |
| DIS | address, count | Disassemble memory | Default count = 10 |
| EDIT | file, drive | Edit file using host editor | — |
| EXPORT | file, drive | Export file to host system | All file types supported |
| FDIR | drive | Full directory listing | Includes deleted files |
| FLOPPY | diskname, drive | Mount disk image | No .disk suffix |
| GO | address, registers | Execute program (jump) | Continue without arguments after break |
| GSB | address, registers | Execute subroutine (JSR) | — |
| IMPORT | file, drive | Import file from host | All file types supported |
| LOAD | file, drive, address | Load file into memory | Default = stored address |
| NEW | file, drive | Create new file | — |
| PACK | drive | Permanently remove deleted files | Frees space on disk |
| PROTECT | file, drive | Protect file from deletion | — |
| PRTB | — | Display breakpoints | — |
| REG | definitions | Display or modify CPU registers | P, S, F, A, X, Y |
| RENAME | file, drive | Rename file | Prompts for new name |
| REVIVE | entry, drive | Restore deleted file | Use FDIR to find entry |
| RESETB | — | Clear all breakpoints | — |
| RUN | file, drive, arguments | Load and execute file | File type must be M |
| SETB | address | Set breakpoint | — |
| STEP | — | Execute one instruction | Continue after break |
| STORE | file, drive, range | Store memory to file | Range = start–end |
| TRACE | count | Execute multiple instructions | Continue after break |
| VOLUME | drive | Set disk label | Does not rename disk |
| / | address | Enter memory inspection mode | Interactive |

Notes:

• Monitor commands require exact file names (no wildcard support)

• File specification format: file[:cyclus][,drive]

• COPY uses memory buffer $2000–$9FFF

• Maximum file size: 32 KB

• LOAD uses stored address by default; optional address overrides it

• After a breakpoint, GO (no arguments), STEP, or TRACE can be used to continue execution

• Multiple versions of a file can be kept on disk (cyclus = version number)

## Pascal Commands

Pascal commands provide the working environment for program development, file handling, graphics, and utilities.

| Command | Arguments | Description | Wildcards |
| --- | --- | --- | --- |
| RUN PASCAL | — | Start Pascal environment | — |
| ARGLIST | arguments | Display program arguments | — |
| BOUNCE | — | Bouncing ball demo | — |
| CALC | — | Calculator (decimal, hex, binary) | — |
| CHKDSK | drive | Check disk directory | — |
| CLEAN | drive | Delete temporary and old files | — |
| COMPILE | file, drive, options | Compile Pascal program | — |
| COMPILE 1 | file, drive, options | Compile pass 1 | — |
| COMPILE 2 | file, drive | Compile pass 2 | — |
| COPY | file, sourcedrive, destdrive | Copy file(s) | Yes |
| DELETE | file, drive | Delete file(s) | Yes |
| DIR | drive or diskname | Display directory | — |
| EDIT | file, drive | Edit file | — |
| NEW | file, drive | Create file for editing | — |
| PACK | drive | Remove deleted files | — |
| FIND | file | Find files | Yes |
| HELP | topic | Display help | Yes |
| GETSOURCE | name | Copy source from PSOURCE | — |
| PUTSOURCE | name | Copy source to PSOURCE | — |
| PUTOBJECT | name | Move object file to PASCAL | — |
| GRAPH | — | Display graph | — |
| GR3D | — | Display 3D graph | — |
| GREND | — | Release graphics memory | — |
| GRTEST | — | Graphics test pattern | — |
| GRTEST2 | — | Alternate test pattern | — |
| PAINT | file, drive | Image editor | Yes |
| SNAP | file | Save graphics snapshot | — |
| SHOW | file | Display image | — |
| TEKTEST | — | Test Tektronix output | — |
| TEK3D | — | 3D Tektronix plot | — |
| TGRAPH | — | Graph on Tektronix | — |
| ERROR | code | Numeric >> text | — |
| EXTIME | file, drive, arguments | Measure execution time | — |
| PCODES | file, startline, endline | Display P-codes | — |
| RESETB | — | Clear breakpoint | — |
| SETB | line | Set breakpoint | — |
| KEYVAL | — | Display key codes | — |
| LEDTEST | — | Test LED display | — |
| MAKETABLE | — | Create sample data table | — |
| SINETST | — | Display sine/cosine graph | — |
| THROWSIM | — | Simulate projectile motion | — |
| WATCH | — | Clock and stopwatch | — |
| VIEW | file | View text file | — |
| TAIL | file | Display end of file | Yes |

Notes:

• File specification format: file[:cyclus][,drive]

• Default working disk is WORK (drive 1)

• Source files are typically stored on PSOURCE

• Wildcards (*, ?) are supported only by commands using WILDLIB

## Error Codes

Error codes are grouped by subsystem and indicate runtime, system, or argument errors.

System Error Codes

| Code | Description |
| --- | --- |
| 01 | Read/write error |
| 02 | Checksum error |
| 03 | Escape exit during read/write |
| 04 | Record number error |
| 05 | File type error |
| 06 | File not found |
| 07 | Disk not ready |
| 08 | Directory full, file not stored |
| 09 | Illegal IRQ |
| 10 | Expression missing |
| 11 | Memory cell cannot be changed |
| 12 | Break table full, not inserted |
| 13 | Illegal memory cell for break |
| 14 | Double breakpoint setting |
| 15 | End of line expected |
| 16 | Syntax wrong in register name or = |
| 17 | Breakpoint not found in table |
| 18 | Syntax wrong in store |
| 19 | File subtype wrong or missing |
| 20 | Wrong file type, not runnable |
| 21 | Unknown monitor command |
| 22 | Illegal opcode for STEP/TRACE |
| 23 | Too many open files, not opened |
| 24 | Direction error in sequential read/write |
| 25 | Wrong file number, file not open |
| 26 | Disk full, file not stored |
| 27 | Random access index out of range |
| 28 | Illegal drive for random access |
| 29 | File not open for random access write |

EXDOS Error Codes

| Code | Description |
| --- | --- |
| 61 | Wildcard not allowed |
| 62 | Only for disk, not tape |
| 63 | Illegal copy |
| 64 | File too large |
| 65 | Write error |
| 66 | Import error |
| 67 | Unknown emulator command |
| 68 | Unable to run mousepad |

Pascal Runtime Error Codes

| Code | Description |
| --- | --- |
| 81 | Division by zero |
| 82 | Stack overflow |
| 83 | Index out of bounds |
| 84 | Program not found or not a Pascal program |
| 85 | Illegal P-code |
| 86 | Escape during execution |
| 87 | No loader file made by compiler |
| 88 | Heap overflow |
| 89 | Pointer not allocated (NIL) |
| 90 | Writing to constant string not allowed |
| 91 | String too long (more than 63 characters) |
| 92 | Only the last allocated string can be released |

Pascal Argument Error Codes

| Code | Description |
| --- | --- |
| 101 | Argument is not string or default |
| 102 | Argument is not number or default |
| 103 | Argument is not starting with / |
| 104 | Unknown argument |
| 105 | Drive is not 0 or 1 |
| 106 | Argument syntax error |
| 107 | Too many arguments |

## Control Keys (Non-printable Characters)

Control functions can be triggered by keyboard input (CTRL combinations) or by writing the corresponding control codes from a program.

Video Control Functions

| Function | Name | Code | CTRL Key | ASCII |
| --- | --- | --- | --- | --- |
| Cursor down | CDOWN | E9 | CTRL-X | 18 |
| Cursor right | CRIGHT | E8 | CTRL-V | 16 |
| Cursor left | CLEFT | E7 | CTRL-C | 03 |
| Cursor up | CUP | E6 | CTRL-Z | 1A |
| Cursor home | CHOME | E5 | CTRL-A | 00 |
| Insert character | INSCHR | E4 | CTRL-U | 15 |
| Delete character | DELCHR | E3 | CTRL-Y | 19 |
| Clear line | CLRLIN | E2 | CTRL-E | 17 |
| Clear to end of screen | CLRDSP | E1 | CTRL-Q | 11 |
| Insert line | INSLIN | C4 | CTRL-D | 04 |
| Delete line | DELLIN | C3 | CTRL-F | 06 |
| Toggle alpha/graphics | TALGRA | E9 | CTRL-L | 0C |
| Escape | ESCAPE | 1B | — | — |
| Set tabulator | SETTAB | 8B | — | — |
| Roll down | RDOWN | 89 | CTRL-B | 02 |
| To right margin | CRMARG | 88 | — | — |
| To left margin | CLMARG | 87 | — | — |
| Roll up | RUP | 86 | CTRL-H | 08 |
| Reverse home | RVHOME | 85 | CTRL-P | 10 |
| Resume writing | RWRITE | 84 | — | — |
| Quiet writing | QWRITE | 83 | — | — |
| Toggle black/white | TBLWHI | 82 | CTRL-E | 05 |
| Clear graphics display | CLRGRA | 81 | — | — |

Emulator Control Keys

| Key | Function |
| --- | --- |
| Shift + ESC | Break (6502 NMI) |
| Shift + MENU | Quit emulator |
| CLOSE WINDOW (x) | Quit emulator |
| Shift + ALT | Shutdown system |

Notes:

• Control functions can be used both via keyboard and program output

• Most functions behave identically in both cases

• Exceptions are rare and usually obvious from the function

## File Types

R65 distinguishes between block (binary) files and sequential (text) files. File types are identified by a subtype suffix.

Block Files (Binary)

| Subtype | Description |
| --- | --- |
| :M | Machine language program |
| :R | Pascal runtime file |
| :X | Other binary file |

Sequential Files (Text)

| Subtype | Description |
| --- | --- |
| :A | Assembler source file |
| :H | Help file |
| :N | NROFF source file |
| :P | Pascal source or library |
| :Q | Pascal loader program |
| :T | Pascal loader library |
| :L | Pascal loader identifier table |
| :W | Pascal compiler reserved words table |
| :B | Other text file |

Notes:

• File subtype is part of the file name

• Block files contain binary data

• Sequential files contain text data

• File handling depends on file type

## How-To — Installation

Install and prepare the R65 emulator, tools, and disks.

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | git clone https://github.com/rricharz/R65 | Download repository |
| 2 | sudo apt-get install libgtk-3-dev | Install dependencies |
| 3 | cd R65 | Enter project directory |
| 4 | make clean; make | Build emulator |
| 5 | cd .. | Return to main directory |
| 6 | cd Tools | Enter tools directory |
| 7 | make clean; make | Build tools |
| 8 | cd .. | Return to main directory |
| 9 | mkdir Files | Create working directory |
| 10 | cd Disks | Enter disk directory |
| 11 | cp EMPTY.disk WORK.disk | Create working disk |

Font Installation

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | cd ~/R65/R65-Emulator/Fonts | Go to font directory |
| 2 | sudo ./install_fonts | Install fonts |

Start Emulator

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | ./emulator | Start from terminal |
| 2 | Desktop icon | Start from desktop |

Disk Initialization

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | FLOPPY WORK,1 | Mount WORK disk |
| 2 | VOLUME 1 | Set disk label |

Notes:

• Disks directory contains disk images

• Files directory is used for host file exchange

• Default setup: PASCAL or PROGRAMS disk in drive 0, WORK disk in drive 1

• Rebuild emulator after updates using make clean; make

## How-To — Working with Disks

Mount disks, set labels, manage files, and maintain disk space using MONITOR

Mount / Switch Disks

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | FLOPPY NAME,drive | Mount disk image NAME to drive |
| 2 | DIR drive | Verify disk contents |

Set Disk Label

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | VOLUME drive | Set or change disk label |
| 2 | enter label | Type label when prompted |

Recommended Setup

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | Drive 0 | PROGRAMS / PASCAL (system disks) |
| 2 | Drive 1 | WORK (your working disk) |

File Operations

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | DIR drive | List files |
| 2 | FDIR drive | List including deleted entries |
| 3 | DELETE file,drive | Delete file |
| 4 | RENAME file,drive | Rename file |
| 5 | PROTECT file,drive | Protect file |
| 6 | REVIVE entry,drive | Restore deleted file |

Space Management

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | PACK drive | Permanently remove deleted files |
| 2 | FDIR drive | Check recoverable space before PACK |

Create New Disks (Host)

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | cp EMPTY.disk NAME.disk | Create new disk image |
| 2 | Restart emulator | Reload disk list |
| 3 | FLOPPY NAME,drive | Mount new disk |
| 4 | VOLUME drive | Set label |

Notes:

• Disk images are stored in the Disks directory

• Use WORK for development; keep system disks unchanged

• Backup important disks before upgrades

• File specification format: file[:cyclus][,drive]

## How-To — Working with Pascal

Typical workflow for editing, compiling, and storing Pascal programs.

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | Drive 0: PASCAL | System disk with compiler and libraries |
| 2 | Drive 1: WORK | Working disk for development |
| 3 | GETSOURCE name | Copy source from PSOURCE to WORK |
| 4 | EDIT or PEDIT name | Edit source file |
| 5 | VIEW name | View source file |
| 6 | COMPILE name | Compile program |
| 7 | name | Run compiled program |
| 8 | PUTSOURCE name | Copy source back to PSOURCE |
| 9 | PUTOBJECT name | Store object file on PASCAL disk |

Automatic Host Interaction

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | EDIT | Stores current file in Files directory |
| 2 | COMPILE | Stores listing in Listing directory |

Notes:

• PSOURCE contains master source files.

• WORK is used for all development and testing

• PASCAL disk contains system and final object files

• File specification format: file[:cyclus][,drive]

## How-To — Printing / printout.txt

Use the printer device to generate output on the host system (printout.txt).

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | Write to @PRINTER | Send output to printer device |
| 2 | Open printout.txt on host | View or process output |
| 3 | Convert to PDF (optional) | Use host tools |
| 4 | Print document | Print from host system |

Typical Usage

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | COMPILE name /options | Generate listing output |
| 2 | Program writes to @PRINTER | Produce formatted output |
| 3 | NROFF input | Format documents for printing |

Screen Printing

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | PRINTON | Send all screen output to printer |
| 2 | Use system normally | All output is captured |
| 3 | PRINTOFF | Stop printing |

Notes:

• Output file is usually named printout.txt

• File is overwritten on each run unless saved externally

• PRINTON/PRINTOFF capture screen output

• NROFF can be used to format manuals and listings

• Printer output is plain text

How-To — Tektronix Graphics

The system uses a Tektronix-compatible terminal or emulator for graphics output.

Attach Tektronix Emulator

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | ./tek4010 -autoclear tail -f printout.txt | Start Tektronix emulator (live output) |
| 2 | ./tek4010 -autoclear cat printout.txt | Display existing output |

Basic Usage

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | Run TEKTEST | Verify Tektronix output |
| 2 | Run TGRAPH | Display graph output |
| 3 | Run TEK3D | Display 3D plot |

From Pascal Programs

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | Use TEKLIB | Include Tektronix library |
| 2 | Call plotting routines | Generate graphics output |
| 3 | Run program | Output appears on Tektronix terminal |

Notes:

• Tektronix emulator reads from printout.txt

• "tail -f" shows live output, "cat" shows existing output

• Graphics output is independent from standard screen output

• TEKLIB provides required routines

## How-To — Editing Files

Choose the appropriate editor for creating, modifying, or inspecting files.

Select Editor

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | EDIT name | External editor (host system) |
| 2 | PEDIT name | Internal full-screen editor |
| 3 | VIEW name | Read-only viewer with search |

Typical Workflow

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | GETSOURCE name | Copy source from PSOURCE to WORK |
| 2 | EDIT or PEDIT name | Modify file |
| 3 | VIEW name | Inspect file |
| 4 | COMPILE name | Compile program |

EDIT (Host Editor)

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | EDIT name | Open file in host editor |
| 2 | Edit and save | Changes written to Files directory |
| 3 | Return to R65 | File is updated automatically |

PEDIT (Internal Editor)

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | PEDIT name | Open file in internal editor |
| 2 | Use editor commands | Modify file directly on disk |
| 3 | Exit editor | Changes are saved |

VIEW (Read-Only)

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | VIEW name | Open file |
| 2 | Scroll / search | Inspect content |
| 3 | Exit VIEW | No changes made |

Notes:

• EDIT uses the host system editor and Files directory

• PEDIT works entirely inside the R65 system

• VIEW is safe for inspection and avoids accidental changes

• Use VIEW for large files and quick navigation

## How-To — Debugging

Debug Pascal programs using line-based breakpoints and execution control.

Prepare Program

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | COMPILE name /L | Compile with line information |

Set Breakpoint

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | SETB line | Set breakpoint at source line |
| 2 | RUN name | Execute program |
| 3 | Break occurs | Program stops at specified line |

Execution Control

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | STEP | Execute next line |
| 2 | TRACE n | Execute multiple lines |
| 3 | GO | Continue execution |

Inspection

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | PCODES line | Display P-codes for line |

Notes:

• Only one breakpoint is supported

• Breakpoints are line-based, not address-based

• Program must be compiled with line option (/L)

• STEP executes line by line

• PCODES shows compiled P-code for a given line

## How-To — File Transfer (Host ↔ R65)

Transfer files between the host system and R65 using IMPORT, EXPORT, and the Files directory.

Export (R65 → Host)

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | EXPORT file,drive | Write file to host (Files directory) |
| 2 | Open Files directory | Access file on host system |

Import (Host → R65)

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | Place file in Files | Prepare file on host system |
| 2 | IMPORT file,drive | Load file into R65 disk |

EDIT Interaction

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | EDIT file | Opens file via host editor |
| 2 | Save file | Stored in Files directory |
| 3 | Return to R65 | File is available on disk |

Typical Workflow

| Step | Command / Action | Description |
| --- | --- | --- |
| 1 | EXPORT file | Move file to host |
| 2 | Edit on host | Use external tools |
| 3 | IMPORT file | Bring file back to R65 |

Notes:

• Files directory is the exchange point between host and R65

• IMPORT and EXPORT support all file types

• Existing files may be overwritten

• File specification format: file[:cyclus][,drive]